# Under the Dome, The Aeon’s Day Recording Notes

## Gearlist

Sequential Circuits Pro-One monosynth

Ensoniq ESQ-1 polysynth

Yamaha TX7 FM polysynth module

Yamaha REX-50 multi-effects module

Yamaha DDS-20M digital delay pedal

| Time (min:sec) | Track(s) | Track Info |
| --- | --- | --- |
| 00:13 | 3 & 4 | Tracks 3 & 4 form a stereo pair, so they should be panned hard-left and hard-right. The sound is a perfect fifth using both oscillators of the Pro-One. The original sound started at around 00:21, but I used long reverb on the REX-50 (recorded backwards) to create the slow fade-in. |
| 00:21 | 1 | ESQ-1 choir pad, fed through REX-50 phaser, then into DDS-20M delay. |
| 00:25 | 2 | Bass drone on the Pro-One with both oscillators in close unison, causing a phasing effect. The low-pass filter has some resonance applied, and the cut-off frequency is ‘played’ in real-time, with LFO square-wave modulation being applied, also. |
| 01:00 | 4 | ‘Bird chirp’ VCS-3 type noises created on Pro-One. Use pan to make the sound ‘fly’ from one side to the other. |
| 01:05 | 3 | Choir notes played on ESQ-1 |
| 03:07 | 3 & 4 | Stereo pair. Low male choir drone with occasional slight pitch bend to add variation. |
| 03:31 | 1 | Octave bass sequence on Pro-One. All sequencers during this passage (tracks 1, 2 & 3) were synced by a click-track which was originally recorded on track 4. Once all three sequencer tracks were recorded, I could free up track 4 to record the lead lines. There isn’t much variation on this sound, but I do wind down the filter envelope depth as it fades out (starting around 08:30). |
| 04:13 | 2 | Higher four-note sequence from Pro-One through digital delay. During the course of this part, I manipulated front-panel controls such as oscillator octaves and waveform. At around 07:44, I even start messing around with the tuning of oscillator B to get those weird high tumbling sounds. |
| 04:55 | 3 | Arpeggiator on Pro-One making abstract noises and complementary sequences through digital delay. I can’t remember if the very high frequency noises were made using the triangle output of Osc B, or whether I used the filter in self-oscillation mode (sine wave). You can certainly hear that the digital delay is creating some aliasing due to not being able to handle the high-frequency components in the input. |
| 04:51 | 4 | Lead lines played on Pro-One (brassy filter sound) and ESQ-1 (WAVBEL preset). The lead lines were recorded after the three sequencer tracks were laid down, as they were using Track 4 for the click track. This part was played in one take, transferring between Pro-One and ESQ-1 as necessary (and occasionally both at once!). The two keyboards were mixed together through digital delay then into reverb. You can sometimes detect a bit of aliasing from the delay. |
| 08:52 | 3 | ESQ-1 choir pad, through Pro-One filter, REX-50 flanger and digital delay! |
| 09:10 | 4 | Resonant sounds from ESQ-1 and fat bass from Pro-One. There seems to be quite a lot going on with the ESQ-1, with both resonant and brassy sounds, so it might have been something recorded into its sequencer rather than played in real-time along with the Pro-One. Or maybe it was just a layered sound - I honestly can’t remember at this stage. |
| 09:28 | 2 | Pro-One making brassy sounds through digital delay and reverb. There seems to be another keyboard with a similar sound which fades in from time to time. I’m guessing that is the ESQ-1, but I have no memory of this. |
| 11:06 | 1 | Autobahn bass sequencer on Pro-One through digital delay. The phasing comes from the two oscillators being very close in frequency. |
| 11:19 | 2 | Autobahn lead line played on ESQ-1 WAVBEL. |
| 11:44 | 4 | Autobahn lead answer on TX7 flute preset. |
| 12:07 | 3 | TX7 log-drum rhythm (played by hand). |
| 13:11 | 2 | Piano preset from ESQ-1. I think the ESQ-1 was the first polysynth to offer a decent piano sound, but I might be wrong. |
| 14:00 | 3 | ESQ-1 strings played through Pro-One filter, REX-50 flanger and digital delay. |
| 16:20 | 3 | Pro-One filter starts closing down so that strings from ESQ-1 form a bassy background drone. |
| 15:14 | 4 | High wood block sounds from TX7 with reverb. |
| 15:38 | 2 | Pan-pipe with pitch bend from TX7 with reverb. |
| 16:59 | 1 | Percussive Pro-One sound through reverb and recorded backwards. |
| 17:05 | 2 | Pro-One four-note sequence played through REX-50 phaser (turned on after 4 bars), then into digital delay. The sequencers (tracks 1, 2 & 3) were all synced to a click track which was originally recorded on Track 4. Once the sequencer tracks were all laid down, Track 4 could then be used to record the lead line. |
| 17:18 | 4 | One of my fave sounds of all time. Pro-One lead uses Osc B hard-sync’ed to Osc A (which is silent). The pitch of Osc B is modulated by LFO, without changing the pitch of Osc A. This makes a lovely shimmery sound, somewhere between pitch mod and PWM. There is a lot of high-frequency information in this sound, which causes a lot of aliasing in the digital delay. I think this adds character to the overall sound, and adds a bit of excitement to the mix. |
| 17:38 | 1 | Pro-One complementary sequencer patterns. I recorded two sequences into memory locations A and B. I could then switch between them at will while recording this take. Unusually for me, there are rests in both these patterns. Rests were a pig to enter into the Pro-One sequencer, as switch bounce from the “Rec/Play” switch would lead to multiple rests being inserted – something you would only discover when you tried auditioning the sequence after recording. It took me multiple attempts to ensure that both sequences were clean. Played through digital delay. |
| 17:45 | 3 | Pro-One making VCS-3 type sweeping chirps in time with sequencers. I think it is Osc B on a triangle wave, rather than the filter in self-oscillation, as there is a lot of aliasing happening with the digital delay. You can sometimes hear the click track breaking through (as a sort of wood block noise). Mains spike at 18:32 might have been my fridge deciding to switch itself on or off during that take. It’s easy to get rid of such things in a DAW, but it was a nightmare trying to remember to pull the fader on that track during mixdown back in the analogue days. |
| 19:00 | 1 | I started messing around with the Pro-One front panel controls to change the nature of the complementary sequencer sound. |
| 20:14 | 4 | LFO modulation of sync’ed Osc B turned up to extreme levels to make ‘puddles’ of noise rather than defined notes. |
| 20:38 | 4 | ESQ-1 organ drone fed through resonant Pro-One filter, modulated by slow triangle wave and fast square wave. Pro-One output fed into digital delay and reverb. |
| 21:17 | 1 & 3 | Pro-One on a Blade Runner style brassy lead sound. The same sound was overdubbed on two tracks to record the question-and-answer lines. It’s nice to pan the two tracks left and right. Very long digital delay on this sound – it must be around 1 second. |

Note that there are some differences between the versions of The Aeon’s Day which appeared on Cochlear Visions CD-R “The Heavens” and the NeuHarmony CD “The Demon-Haunted World”.

Firstly, the lead line played on Track 4 at 04:51. The original version used a melody from the advertising 7” single The Cimex Invasion (I’ve uploaded a recording of this to YouTube). I had always wanted to do a tribute to this single, which meant a lot to me when I was growing up, but Dave Law was strict about having no cover versions on NeuHarmony releases. I overdubbed the WAVBEL bits in Pro-Tools, with a similar-but-just-different-enough melody to avoid this problem. Obviously, the cassette multi-track was not affected by this edit!

The Autobahn tribute was obviously out of the question, so I created an entirely new section which used the sequencer backing from Strange Attractor as its basis. I think it comes out sounding quite a bit like Deep Distance (from New Age of Earth by Ashra).